## Torque Values for XPAG Engines

### Engine

| Description | Torque |
| --- | --- |
| GUDGEON PIN CLAMP BOLT | 25 LBFT |
| ROD BIG END BOLTS | 25 LBFT |
| MAIN BEARING CAP NUTS | 63 LBFT |
| FLYWHEEL TO CRANKSHAFT BOLTS | 50 LBFT |
| CLUTCH PRESSURE PLATE BOLTS | 19 LBFT |
| OIL PICKUP PIPE TO SUMP BOLTS | 6 LBFT |
| CAMSHAFT LOCATING PLATE BOLTS | 6 LBFT |
| CAMSHAFT SPROCKET BOLT | 50 LBFT |
| TIMING CHAIN TENSIONER BOLTS | 6 LBFT |
| SUMP TO BLOCK BOLTS | 19 LBFT |
| TIMING CHAIN COVER BOLTS | 19 LBFT |
| CRANKSHAFT PULLEY BOLT | 50 LBFT |
| CYLINDER HEAD NUTS | 50 LBFT |
| ROCKER TOWER BOLTS (8 MM) | 16 LBFT |
| ROCKER TOWER BOLTS (10 MM) | 43 LBFT |
| OIL PUMP TO BLOCK BOLTS | 6 LBFT |

### Other

| Description | Torque |
| --- | --- |
| WATER PUMP PULLEY NUT | 10 LBFT |
| GENERATOR PULLEY NUT | 35 LBFT |
| TRANSMISSION OUTPUT FLANGE NUT | 65 LBFT |
| REAR AXEL HUB NUT | 170 - 200 LBFT |
| FRONT AXEL HUB NUT | 40 - 70 LBFT |
| WHEEL LUG NUTS | 50 LBFT |